# Vivado User Guide

# Start by creating a project

# Choose where to save the project and a name for it

The project type must be RTL Project

# You can add the RTL files now or after the project is created. We will add the RTL files now.

# Skip the Add Constraints step

# At this step select Boards and choose the first one (this step can also be skipped without selecting a board ☺)

# Add the verification files (.sv etc.): select Simulation sources and click +

# Load the files

# Run a simulation by selecting Run Simulation

# To collect coverage, open the settings from Edit -> Settings and fill them in as shown in the images.

The values to set:

Compilation tab: xsim.compile.xvlog.more_options = -L uvm

Elaboration tab: xsim.elaborate.xelab.more_options = -L uvm

Elaboration tab: xsim.elaborate.coverage.name = cov_report, xsim.elaborate.coverage.dir = ./, xsim.elaborate.coverage.type = sbct (all coverage types)

# In the Tcl Console, type the following command:

# export_xsim_coverage -cov_db_name cov_report -cov_db_dir ./ -output_dir cReport -open_html true

# The coverage report can be found at the following path.

# The coverage report is an HTML page

# Using this guide with template_with_checker

When you add the simulation sources, add only testbench.sv from the template_with_checker folder: it includes all the other files of the template. If Vivado cannot find the included files, add the template_with_checker folder to the include search paths (Settings -> General -> Verilog options).

Set Simulation top module name to top (the module in testbench.sv), not to the DUT.

In the Simulation tab, set xsim.simulate.runtime to -all, so the test runs until UVM ends it; the default stops the simulation after 1000 ns.

At the end of the log you should see the coverage values, the Checker summary and TEST PASS or TEST FAIL.